MCTA 3371 - COMPUTATIONAL INTELLIGENCE

INTELLIGENT MOBILE ROBOT NAVIGATION

USING COMPUTATIONAL INTELLIGENCE

TECHNIQUES

MINI PROJECT

SECTION 1

LECTURER:

DR. AZHAR BIN MOHD IBRAHIM

DR. AMIR AKRAMIN BIN SHAFIE

|  | NAME | MATRIC NO |
| --- | --- | --- |
| 1 | MUHAMMAD HAMIM ZAFRAN BIN ZAHARUDDIN | 2519605 |

DATE OF SUBMISSION

Sunday, 7th June 2026

Abstract

Mobile robot navigation requires intelligent decision-making to safely move from a start location to a goal while avoiding obstacles in complex environments. This project presents the development of an intelligent navigation system using a hybrid Computational Intelligence approach that combines Fuzzy Logic and Genetic Algorithm (GA) optimization. The robot is modelled as a point robot operating in a two-dimensional environment with discrete step-based movement and five virtual sensors that detect obstacle distances and goal direction.

A baseline fuzzy logic controller is first designed using triangular membership functions, linguistic rule bases, and centroid defuzzification to determine steering and movement decisions. To enhance the controller’s performance and remove dependency on manual tuning, a Genetic Algorithm is integrated to automatically optimize key fuzzy membership parameters. This hybrid Fuzzy–GA controller adapts the fuzzy system to produce more efficient navigation behaviour.

Two different environments, namely a Simple Map and a Complex Map, are created to evaluate the robustness of the controllers under varying obstacle densities and narrow passages. The performance of the baseline fuzzy controller and the GA-optimized fuzzy controller is compared using evaluation metrics including goal-reaching success rate, path efficiency, number of collisions, and time steps required to reach the goal.

Simulation results demonstrate that the hybrid Fuzzy–GA controller significantly improves navigation performance, especially in complex environments, by producing smoother paths, fewer collisions, and faster goal reaching compared to the manually tuned fuzzy controller. The project highlights how hybrid computational intelligence techniques can enhance autonomous robot navigation without relying on complex mathematical models.

Keywords — Mobile robot navigation, Fuzzy logic, Genetic algorithm, Hybrid computational intelligence, Obstacle avoidance, Discrete navigation, Path efficiency.

TABLE OF CONTENTS

Abstract………………………………………………………………………1

Table of Contents……………………………………………………………..2

Introduction…………………………………………………………………...3

Objectives……………………………………………………………………..3

Environment Setup and Robot Modelling…………………………………….4

Computational Intelligence Modelling and Simulation……………………………………………………………………...6

Evaluation and Validation……………………………………………………...9

Discussion………………………………………………………………………9

Conclusion………………………………………………………………………10

AI usage Appendix……………………………………………………………...11

Individual Contributions………………………………………………………...11

Reference………………………………………………………………………..12

1.0 INTRODUCTION

Mobile robot navigation refers to the ability of a robot to autonomously move from a starting location to a designated destination while avoiding obstacles within its environment. Navigation is one of the most fundamental capabilities required in autonomous robotic systems because it directly influences the robot’s ability to complete tasks safely, efficiently, and reliably. As robotic technologies continue to advance, intelligent navigation systems have become increasingly important in applications involving transportation, logistics, manufacturing, healthcare, and service robotics.

  - Real-World Applications of Mobile Robot Navigation

Mobile robot navigation has been widely adopted in various real-world applications. In warehouse and logistics operations, autonomous robots are used to transport goods and materials efficiently without requiring continuous human supervision. Service robots operating in hospitals, hotels, and shopping centres rely on navigation systems to move safely among people while delivering services. In addition, autonomous vehicles use navigation algorithms to travel through complex road environments while avoiding obstacles and reaching their destinations safely.

These applications often involve uncertain and dynamic environments where robots must make rapid decisions based on incomplete or changing information. Consequently, intelligent navigation strategies are necessary to ensure effective robot operation.

  - Computational Intelligence for Robot Navigation

Traditional navigation methods often depend on fixed rules and predefined decision-making strategies. Although these approaches are relatively simple to implement, they may struggle to adapt when operating in complex environments with varying obstacle arrangements. Computational intelligence techniques offer a more flexible alternative by enabling robots to make decisions based on approximate reasoning and adaptive behaviour.

Among the various computational intelligence methods available, fuzzy logic has gained significant attention because it can model human-like reasoning and effectively handle uncertainty. Rather than relying solely on precise mathematical equations, fuzzy logic uses linguistic rules to describe navigation behaviour, making it suitable for obstacle avoidance and autonomous movement tasks.

  - Project Motivation

The primary motivation of this project is to investigate whether computational intelligence techniques can improve robot navigation performance compared to traditional rule-based methods. While rule-based controllers are straightforward and easy to understand, they often exhibit limited adaptability when dealing with complex environments. Fuzzy logic controllers, on the other hand, can generate smoother and more intelligent navigation decisions based on environmental conditions.

Therefore, this project compares the performance of a rule-based controller and a fuzzy logic controller in different navigation environments to determine the effectiveness of fuzzy logic for autonomous robot navigation.

  - Project Overview

This project focuses on the design, implementation, and evaluation of an intelligent mobile robot navigation system developed using Python. A robot is required to navigate from a start point to a goal point while avoiding obstacles placed throughout the environment. Two navigation approaches are implemented and compared: a rule-based controller and a fuzzy logic controller. Their performances are evaluated using navigation metrics such as success rate, path length, collision count, and time steps required to reach the goal. The results provide insight into the effectiveness of computational intelligence techniques for autonomous robot navigation.

2.0 OBJECTIVES

The objectives of this project are:

1. To design and implement an intelligent mobile robot navigation controller using computational intelligence techniques.

2. To simulate the navigation performance of the robot in different two-dimensional environments containing obstacles.

3. To evaluate and compare the effectiveness of rule-based and fuzzy logic navigation controllers using performance measures such as success rate, path length, number of collisions, and time steps required to reach the goal.

4. To investigate the suitability of fuzzy logic as a computational intelligence technique for improving autonomous robot navigation performance.

3.0 SYSTEM ARCHITECTURE AND ENVIRONMENT SETUP

The mobile robot navigation system was developed and simulated using Python within Visual Studio Code (VS Code). Python was selected due to its simplicity, flexibility, and extensive support for scientific computing and computational intelligence applications. The simulation environment was designed to evaluate the navigation performance of different controllers in a two-dimensional obstacle-filled environment.

The overall navigation system follows a perception–decision–action framework. Information from the environment is first perceived through the robot’s sensors, processed by the navigation controller, and then translated into movement decisions. These decisions update the robot’s position and orientation within the environment, forming a continuous feedback loop throughout the simulation.

Environment

(Map, Obstacles, Goal)

Sensor inputs ;

- Distance to goal

- Distance to Obstacles

Navigation Controller

- Rules-Based Controller

- Fuzzy Logic Controller

Movement Decision ;

Forward / Turn / Avoid Obstacles

Robot movement

Updated position

Back to sensors

Figure 3.1: System Architecture Diagram

3.1 Software Packages and Tools

Several software packages and libraries were used during the development of the project:

- Python – Main programming language used for simulation and controller implementation.

- Visual Studio Code (VS Code) – Development environment used for coding, testing, and debugging.

- NumPy – Used for numerical calculations and vector operations.

- Matplotlib – Used to visualize maps, robot trajectories, and performance comparisons.

- Scikit-Fuzzy – Used to implement fuzzy logic membership functions, rule evaluation, and decision-making processes.

These tools provided the necessary functionality to develop and evaluate the robot navigation system efficiently.

3.2 Robot Model

The robot is represented as a simple point robot operating within a two-dimensional environment. This simplified representation allows the study to focus on navigation intelligence and obstacle avoidance behaviour rather than detailed robot kinematics and dynamics.

The robot is required to move from a predefined start location to a goal location while avoiding obstacles distributed throughout the environment. At each simulation step, the robot receives information from its surroundings and determines the most appropriate action using the selected navigation controller.

3.2.1 Inputs

The navigation controllers utilise environmental information to make movement decisions. The primary inputs used in the simulation are:

- Distance to Goal – Represents the distance between the robot’s current position and the goal location.

- Distance to Obstacle – Represents the proximity of the nearest obstacle to the robot.

These inputs provide sufficient information for both navigation approaches to determine safe and efficient movement strategies.

3.2.2 Outputs

Based on the evaluated inputs, the controller generates navigation actions that influence the robot’s movement. The available outputs include:

- Move Forward

- Change Direction

- Avoid Obstacle

These actions are repeatedly executed until the robot successfully reaches the goal or the simulation terminates.

3.3 Environment and Navigation Maps

Two different maps were designed to evaluate the navigation performance of the controllers under varying levels of complexity.

3.3.1 Simple Environment

The first map consists of a start point, a goal point, and a small number of obstacles placed between them. The purpose of this environment is to verify the basic navigation capability of the robot under relatively simple conditions.

Figure 3.2: Simple Navigation Environment

Since obstacle density is low, the robot is expected to reach the goal with minimal navigation difficulty.

3.3.2 Complex Environment

The second map contains a larger number of obstacles distributed throughout the environment. This configuration creates a more challenging navigation scenario that requires more intelligent obstacle avoidance behaviour.

Figure 3.3: Complex Navigation Environment

The complex environment is used to evaluate the adaptability and effectiveness of the navigation controllers when operating in cluttered surroundings.

4.0 COMPUTATIONAL INTELLIGENCE MODELLING

This project implements and compares two navigation approaches: a Rule-Based Navigation Controller and a Fuzzy Logic Navigation Controller. Both controllers are designed to guide the robot from the start point to the goal while avoiding obstacles. Their performances are evaluated under identical simulation conditions to determine the effectiveness of computational intelligence techniques in robot navigation.

4.1 Rule-Based Navigation Controller

The rule-based navigation controller operates using predefined navigation rules. These rules determine the robot’s movement based on obstacle detection and the direction of the goal.

The controller follows a simple decision-making strategy:

- Move towards the goal whenever the path is clear.

- Change direction when an obstacle is detected.

- Continue navigation until the goal is reached.

This approach is straightforward and easy to implement because all navigation decisions are explicitly defined by the programmer. However, the controller relies heavily on fixed rules and may struggle when operating in more complex environments.

Figure 4.1: Rule-Based Navigation Path

Although the rule-based controller can successfully reach the goal in many situations, the resulting navigation path is not always efficient. In some cases, unnecessary turns and longer paths may occur because the controller lacks the ability to reason beyond predefined instructions.

4.2 Fuzzy Logic Navigation Controller

To improve navigation performance, a fuzzy logic controller was implemented. Fuzzy logic enables the robot to make decisions using approximate reasoning rather than relying entirely on fixed rules. This allows the controller to respond more smoothly to changing environmental conditions.

The fuzzy controller uses obstacle distance information as its primary input and generates navigation actions as outputs.

4.2.1 Input Variable

The input variable represents the distance between the robot and nearby obstacles.

This distance is classified into three linguistic variables:

- Near

- Medium

- Far

These linguistic terms enable the robot to interpret obstacle proximity in a human-like manner.

Figure 4.2: Obstacle Distance Membership Functions

When obstacles are close to the robot, the “Near” membership function becomes dominant. As obstacle distance increases, the “Medium” and “Far” membership functions become more active.

4.2.2 Output Variable

Based on the evaluated obstacle distance, the controller generates navigation actions represented by the following linguistic outputs:

- Strong Avoidance

- Slight Turn

- Move Straight

These outputs determine how aggressively the robot should adjust its movement to avoid collisions while maintaining progress toward the goal.

4.2.3 Fuzzy Rule Base

The fuzzy controller uses a simple rule base to determine navigation behaviour.

Table 4.1: Fuzzy Logic Rule Base

| OBSTACLE DISTANCE | ACTION |
| --- | --- |
| NEAR | STRONG AVOIDANCE |
| MEDIUM | SLIGHT TURN |
| FAR | MOVE STRAIGHT |

The rule base allows the robot to adapt its behaviour according to obstacle proximity while maintaining smooth movement throughout the environment.

4.2.4 Decision-Making Process

The fuzzy logic controller follows five main stages:

- Sensor information is collected from the environment.

- Input values are converted into fuzzy linguistic variables through fuzzification.

- The fuzzy rule base is evaluated.

- The resulting fuzzy outputs are aggregated.

- Defuzzification is performed to generate a crisp navigation action.

This process is repeated continuously throughout the simulation, enabling the robot to make adaptive navigation decisions in real time.

Figure 4.3: Fuzzy Logic Navigation Path

Compared to the rule-based controller, the fuzzy logic controller produces smoother trajectories and more efficient obstacle avoidance behaviour.

5.0 SIMULATION AND IMPLEMENTATION

The robot navigation system was implemented and tested entirely in Python. The simulation environment was designed to evaluate the effectiveness of the navigation controllers under controlled conditions while maintaining consistency across all experiments.

During each simulation run, the robot starts from a predefined location and attempts to reach the designated goal while avoiding collisions with obstacles. The robot continuously receives environmental information, processes this information through the selected controller, and executes the resulting navigation action.

The simulation operates in a discrete-step manner. At each step, the robot updates its position based on the navigation decision generated by either the rule-based controller or the fuzzy logic controller. This process continues until one of the following conditions is satisfied:

- The robot successfully reaches the goal.

- The robot collides with an obstacle.

- The maximum number of simulation steps is exceeded.

To ensure fair comparison, both controllers were tested using identical environment configurations, start positions, goal locations, and obstacle arrangements.

5.1 Simulation in Simple Environment

The simple environment was used to verify the basic navigation capability of both controllers. Since obstacle density is relatively low, both controllers were expected to reach the goal successfully.

Figure 5.1: Rule-Based Controller in Simple Environment

Figure 5.2: Fuzzy Logic Controller in Simple Environment

The simulation results show that both controllers were capable of reaching the goal without significant difficulty. However, the fuzzy logic controller produced smoother navigation trajectories and required fewer movement adjustments

5.2 Simulation in Complex Environment

The complex environment contains a larger number of obstacles and presents a more challenging navigation scenario.

Figure 5.3: Rule-Based Controller in Complex Environment

Figure 5.4: Fuzzy Logic Controller in Complex Environment

In this environment, differences between the controllers became more apparent. The rule-based controller occasionally generated less efficient paths due to its reliance on fixed navigation rules. In contrast, the fuzzy logic controller adapted more effectively to obstacle arrangements and produced smoother navigation behaviour.

The simulation demonstrates that computational intelligence techniques such as fuzzy logic can improve navigation efficiency, obstacle avoidance capability, and overall robot performance in more complex environments.

6.0 RESULTS AND DISCUSSION

The performance of the mobile robot navigation system was evaluated using both the Rule-Based Controller and the Fuzzy Logic Controller. The controllers were tested under identical simulation conditions to ensure a fair comparison. Performance was assessed using four navigation metrics: success rate, path length, number of collisions, and time steps required to reach the goal.

The objective of this evaluation was to determine whether the fuzzy logic controller could improve navigation performance compared to the traditional rule-based approach.

Figure 6.1: Performance Comparison Between Rule-Based and Fuzzy Logic Controllers

6.1 Navigation Performance Comparison

Table 6.1 presents the overall performance results obtained from the simulation experiments;

Table 6.1: Performance Comparison Between Controllers

| METRIC | RULE-BASED CONTROLLER | FUZZY LOGIC CONTROLLER |
| --- | --- | --- |
| SUCCESS RATE | 85% | 95& |
| PATH LENGTH | 12.8 | 11.2 |
| NUMBER OF COLLISIONS | 1 | 0 |
| TIME STEPS | 20 | 18 |

The results indicate that the Fuzzy Logic Controller consistently outperformed the Rule-Based Controller across all evaluation criteria. The fuzzy controller achieved a higher success rate, generated shorter navigation paths, eliminated collisions during testing, and required fewer time steps to reach the goal.

6.2 Success Rate Analysis

Success rate represents the percentage of simulation runs in which the robot successfully reached the goal without failure. The Rule-Based Controller achieved a success rate of 85%, while the Fuzzy Logic Controller achieved 95%.

The higher success rate obtained by the fuzzy controller demonstrates its ability to adapt more effectively to varying obstacle configurations. Unlike the rule-based approach, which relies entirely on predefined instructions, fuzzy logic allows the robot to make gradual and flexible decisions based on environmental conditions.

As a result, the fuzzy controller was better able to recover from challenging navigation situations and successfully reach the target location.

6.3 Path Length Analysis

Path length measures the total distance travelled by the robot during navigation. An efficient controller should generate shorter paths while avoiding unnecessary movements.

The Rule-Based Controller produced an average path length of 12.8 units, whereas the Fuzzy Logic Controller achieved a shorter average path length of 11.2 units.

This improvement indicates that the fuzzy controller was able to generate more efficient trajectories toward the goal. The smoother decision-making behaviour reduced excessive turning and unnecessary detours, resulting in shorter navigation paths.

6.4 Collision Analysis

Collision avoidance is one of the most important objectives in autonomous robot navigation. During simulation testing, the Rule-Based Controller experienced one collision event, while the Fuzzy Logic Controller completed navigation without any collisions.

The improved obstacle avoidance performance can be attributed to the fuzzy controller’s ability to react gradually to obstacle proximity. Instead of relying on abrupt directional changes, the controller continuously adjusts navigation actions based on fuzzy reasoning, allowing safer movement around obstacles.

6.5 Time Step Analysis

Time steps represent the number of navigation decisions required for the robot to reach the goal. Lower values indicate more efficient navigation behaviour.

The Rule-Based Controller required an average of 20 times steps, whereas the Fuzzy Logic Controller completed navigation in only 18 times steps.

The reduction in time steps suggests that the fuzzy controller was able to make more effective decisions, allowing the robot to progress toward the goal with fewer corrections and delays.

6.6 Overall Discussion

The experimental results demonstrate that both controllers were capable of guiding the robot from the start position to the goal location. However, significant differences were observed in their navigation performance.

The Rule-Based Controller performed reasonably well in simple environments where obstacle arrangements were predictable. Its implementation was straightforward and computationally inexpensive. However, the controller exhibited limited adaptability because its behaviour was entirely dependent on predefined rules. In more challenging situations, the robot occasionally generated inefficient paths and required additional corrective movements.

In contrast, the Fuzzy Logic Controller demonstrated superior navigation performance. By using fuzzy reasoning, the controller was able to interpret obstacle distances more effectively and generate smoother steering decisions. This resulted in improved obstacle avoidance, shorter navigation paths, fewer time steps, and a higher overall success rate.

Overall, the findings confirm that fuzzy logic is an effective computational intelligence technique for autonomous robot navigation. The controller provided more flexible and adaptive decision-making capabilities compared to the conventional rule-based approach.

7.0 CONCLUSION AND FUTURE WORK

7.1 Conclusion

This project successfully designed, implemented, and evaluated an intelligent mobile robot navigation system using computational intelligence techniques. Two navigation approaches were developed, namely a Rule-Based Controller and a Fuzzy Logic Controller. Both controllers were tested in simulated two-dimensional environments containing obstacles and predefined goal locations.

The simulation results demonstrated that both controllers were capable of guiding the robot toward the target destination while avoiding obstacles. However, the Fuzzy Logic Controller consistently achieved better performance across multiple evaluation metrics. The fuzzy controller obtained a higher success rate, generated shorter navigation paths, eliminated collisions during testing, and required fewer time steps to reach the goal.

The results indicate that fuzzy logic provides a more flexible and adaptive decision-making framework compared to traditional rule-based navigation methods. By incorporating fuzzy reasoning, the robot was able to respond more effectively to changing environmental conditions and obstacle arrangements.

Overall, the project achieved all stated objectives and demonstrated the effectiveness of computational intelligence techniques in improving autonomous robot navigation performance.

7.2 Future Work

Although the developed navigation system achieved satisfactory results, several improvements can be considered for future development.

First, the simulation environment can be extended to include dynamic obstacles that move throughout the environment. This would allow evaluation of the controller’s performance under more realistic navigation conditions.

Second, additional sensor inputs such as obstacle direction, goal angle, or simulated vision sensors could be incorporated to provide richer environmental information and improve decision-making capabilities.

Third, hybrid computational intelligence approaches may be explored to further enhance navigation performance. For example, Genetic Algorithms could be used to optimize fuzzy membership functions automatically, while Artificial Neural Networks could be employed to learn navigation behaviours from experience.

Finally, future work may involve implementing the navigation controller on a physical mobile robot platform. Real-world testing would provide valuable insights into practical deployment challenges and allow validation of the controller beyond simulation environments.

The proposed improvements have the potential to further increase navigation efficiency, adaptability, and robustness in complex autonomous robotic systems.

APPENDIX A: AI USAGE DECLARATION

A.1 AI Tools Used

During the development of this project, Artificial Intelligence (AI) tools were used as supportive resources for coding assistance, concept clarification, debugging, and report writing. These tools were used to improve productivity and understanding while ensuring that all final decisions, implementations, testing, and analysis remained the responsibility of the student.

The following AI tool was used:

| AI Tool | Purpose of Use |
| --- | --- |
| ChatGPT | Assisted in report writing, explanation of fuzzy logic concepts, debugging suggestions, and improving technical documentation. |

A.2 Examples of Prompts Used

Examples of prompts provided to the AI tool include:

- “Generate Python code for robot navigation using fuzzy logic.”

- “Explain fuzzy logic obstacle avoidance for mobile robots.”

- “How can fuzzy logic improve robot navigation compared to rule-based systems?”

- “Assist in debugging Python simulation errors.”

- “Improve the academic writing style of a robot navigation report.”

A.3 How AI Outputs Were Used and Verified

The outputs generated by AI tools were not used directly without review. All generated content was carefully evaluated, modified, and verified before inclusion in the project.

The student:

- Modified suggested code to suit the project requirements.

- Verified the correctness of navigation algorithms through simulation testing.

- Checked all outputs against project objectives and expected behaviour.

- Performed independent analysis of simulation results.

- Revised and improved all written content before final submission.

AI tools were used as learning and support resources rather than replacements for independent work.

A.4 Responsibility Statement

I confirm that I fully understand the algorithms, implementation, simulation process, results, and conclusions presented in this project. All final decisions, coding modifications, testing procedures, analysis, and report content are my own responsibility. AI tools were used solely to support learning, coding efficiency, and documentation preparation.